# HSP14CS-V2 Firmware Update Instructions

Step 1 – Connect the Device

Connect the UPGRADE port of the HSP14CS-V2 to your PC using a micro-USB cable.

Step 2 – Power On and Identify COM Port

Power on the HSP14CS-V2. Open Device Manager on your PC and identify the correct COM port assigned to the device.

Step 3 – Open Update Tool

Launch the Blustream Update Manager tool on your PC.

In the Connection section:

- Select the COM connection option.

- Choose the correct COM port identified earlier.

- Set the baud rate to 57600.

- Ensure the 13 Bytes option is checked.

Click Connect.

The button will change from Connect to Disconnect once the connection is established.

Step 4 – Load Firmware File

In the Upgrade section:

- Click the blue Open button.

- In the pop-up window, browse to and select the correct firmware file.

- Click Open to load the file.

Step 5 – Start Firmware Upgrade

Click Upgrade to begin the firmware update process.

- The progress bar will indicate the update status and completion percentage.

Step 6 – Complete Update

Once the update is finished, a pop-up message will display:

“Upgrade successfully!”

Close the Blustream Update Manager tool to complete the process.

.